Polymer Anti-Mouse/Rabbit HRP-DAB

Dual Kit

ORDER INFORMATION

Name of Products:	Polymer Plus Anti Mouse/Rabbit HRP-DAB Dual Kit

Catalogue Number: 	nAP-10210-1

Size:			11 ml kit (w DAB chromogen and buffer) for 100 slides

Storage:		4-8°C

Intended Use:

Polymer Anti-Mouse/Rabbit HRP-DAB Dual Kit is a HRP polymer based detection system that can be used to detect mouse or rabbit primary antibodies for immunohistochemical staining of human tissues. Polymer kits are the first generation polymer detection system that uses polymeric HRP-linked secondary antibody conjugates to get consistent immunostaining outcomes on archival tissues and to increase sensitivity on difficult-to-work antibodies. This technology provides excellent sensitivity and high specificity. It is a biotin-free system, therefore, overcomes the non-specific staining caused by streptavidin/biotin system due to endogenous biotin. This kit has been tested on paraffin embedded tissue only; however it may be used on frozen or freshly prepared monolayer cell smears.

Kit components:

| Components | Name | Volume |
| --- | --- | --- |
| Reagent 1 | Polymer HRP anti-Mouse/Rabbit IgG (Ready To Use). | 11ml |
| Reagent 2A | DAB Substrate (Ready To Use) | 20ml |
| Reagent 2B | DAB Chromogen (20x) | 1ml |

Recommended Protocol:

1. Fixation: To ensure the quality of the staining and obtain reproducible performance, user needs to supply appropriately fixed tissue and well prepared slides.

2. Tissue needs to be adhered to the slide tightly to avoid tissue falling off.

3. Paraffin embedded section must be deparaffinized with xylene and rehydrated with a graded series of ethanol before staining.

4. Cell smear samples should be made as much monolayer as possible to obtain satisfactory results.

5. Investigator needs to optimize dilution and incubation times for primary antibodies.

6. Three control slides will aid the interpretation of the result: positive tissue control, reagent control (slides treated with Isotype control reagent), and negative control.

7. Staining steps: DO NOT let specimen or tissue dry from this point on.

8. Serum blocking before primary antibody incubation for AP’s Polymer and Polymer Plus is not required because all our antibody conjugates are absorbed to human serum.

| Reagent | Staining Procedure | Incubation Time |
| --- | --- | --- |
| 1. PEROXIDASE BLOCKING REAGENT. Supplied by user | a. Incubate slides in PEROXIDASE BLOCKING REAGENT (Ready-to-use 3% H2O2 solution) for 10 minutes. b. Rinse the slide using distilled water. | 10min |
| 2. HIER PRETREATMENT: | a. Heat Induced Epitope Retrieval (HIER) may be required for primary antibody suggested by vendor. Please check the data sheet of primary antibody. b. Wash with PBS-T (PBS containing 0.05% Tween-20) 3 times for 2 minutes each time. |  |
| 3. PRE-BLOCK (Optional) Not provided | a. Add 2 drops (100μL) or enough volume of 10% Normal Goat Serum (nAP-20001) to completely cover the tissue section, Incubate for 10 min. b. Drain or blot off solution. DO NOT RINSE c. See note 8 in Recommended Protocol. | 10min |
| 4. PRIMARY ANTIBODY Supplied by user | a. Apply 2 drops (100μL) or enough volume of PRIMARY ANTIBODY to cover the tissue section completely. Incubate in moist chamber for 30-60 min. b. Wash with PBS-T (PBS containing 0.05% Tween-20) 3 times for 2 minutes each time. | 30-60min |
| 5. Reagent 1 Polymer HRP anti-Mouse/Rabbit IgG (RTU) | a. Apply 2 drops (100μL) or enough volume of Reagent 2 Polymer HRP anti- Mouse/Rabbit IgG to cover each section. Incubate in moist chamber for 15-20 min. | 15-30min |
|  | b. Wash with PBS-T (PBS containing 0.05% Tween-20) 3 times for 2 minutes each time. |  |
| 6. CHROMOGEN Reagent 3A: DAB Substrate(RTU) Reagent 3B: DAB Chromogen (20x) | a. Add 1 drop or 2 drops (for higher sensitivity and contrast) of Reagent 3B into 1mL Reagent 3A. Mix well. b. Apply 2 drops (100μL) or enough of DAB working solution to completely cover tissue section. Incubate for about 5 min. Monitor the color development under microscope. c. Rinse well in distill or tap water Note: Protect DAB working solution from light and use within 5 hours. | 5min |
| 7. HEMATOXYLIN Supplied by user | a. Counterstain with 2 drops (100μL) or enough volume of Hematoxylin to cover tissue completely and wait about 20-30 seconds. 20 seconds b. Rinse well with running tap water for 1-2 minutes. c. Put slides in PBS until show blue color (about 30-60 seconds). d. Rinse well in distill or tap water | 20 Second |
| 8. Mounting Medium: Supplied by user | Follow the manufacture data sheet procedure for mounting. Recommended product: Angio-Proteomie: Cat. No. nAP-20002 | Refer to data sheet |

Precautious:

DAB may be carcinogenic. Please wear gloves and take other necessary precautions.

Remarks:

For Research Use only

Caution: Handling human tissue derived products is potentially bio-hazardous, always wear gloves and safety glasses when working on these materials. Never mouth pipette. We recommend following the universal procedures for handling products of human origin as the minimum precaution against contamination.